Ways to run LOGISNET

LOGISNET can be running with two sources of inputs

- The default inputs are the calculation of Pit filled DEM, flow direction, slope and, contributory area made them with ARCINFO resources.

- As an alternative source of inputs are the Pit filled DEM, flow direction, slope and, contributory area made them with  SINMAP model resources.

The only difference between the two inputs sources is the way that is calculated. ArcInfo use an eight neighborhood cell method to calculate

| D8 or eight neighborhood is calcuated from the 3x3 neighborhood using the average maximum technique (see references). -These flow directions are coded with integers (e.g., 1 for east, 2 for southeast, 4 for south). -“The Eight Direction Pour Point Model is reasonably accurate. Researchers have come up with several alternatives to the Eight Direction Pour Point Model that more closely represent how water actually flows across the terrain. Unfortunately they are more complicated to apply and usually have similar results to the Eight Direction Pour Point Model.” Advanges and Disadvanges: 1)At the edge of the grid, at least three cells (outside the grid’s extent) will contain nodata as their z values. These cells will be assigned the center cell’s z value. The result is a flattening of the 3x3 plane that is fit to these edge cells, 2) The D8 method does well on points 1, 4, and 5 of Dά advantages metghod, but resolves flow directions too coarsely (point 3), introducing grid bias (point 2). | SINMAP: D These flow directions are coded with float as an angle. Advantages: “(1) the need to avoid or minimize dispersion; (2) the need to avoid grid bias, due to the orientation of the numerical grid; (3) the precision with which flow directions are resolved; (4) a simple and efficient grid based matrix storage structure; and (5) robustness, the ability to cope with “difficult” data, such as the saddle in Figure 1 but also with pits and flat areas. Disadvanges: 1) Increase loose of information at the border in the grid input sources. |
| --- | --- |

LOGISNET can run with both sources. In order to switch between this two souces, the user have to choose form the menu :

| Step 1. Select the opcion Run as SINMAP or as LOGISNET inputs opcion. | Step2. Select the input sources. |
| --- | --- |

The only thin that this option do is if SINMAP inputs are selected is:

- activate a variable called .sinmaprun. tha is used in the code of si1.aml si1mod.aml, si1mod.aml, si1perpoint1.aml,  si1perbed1.aml and si1perbed2.aml that state how to take in account the slope map. The code looks like:

&select %.sinmaprun%

&when LOGISNET

&do

s = (slope(..\%.sinmapdem2%fel) div DEG)                   /* Slope en radianes.

&end

&when SINMAP

&do

s = atan(..\%.sinmapdem2%slp)

&end

&end

- Make a copy of the contributory area from nameSCA  grid map  to nameAREA grid map.

- Just in case that you want to run the light version of SINMAP in LOGISNET, this module also allow to set the variables .waterdensity = 1000. 0 and .soildensity = 2000.0  required by SINMAP light.

An example of outputs of LOGISNET and SINMAP

1. The calibration parameter with 4 regions:

The outputs showed here are a) SINMAP  b)LOGISNET with SINMAP inputs and c) LOGISNET with ArcInfo Inputs (default)

| 1)SINMAP. | b)LOGISNET with SINMAP | ) LOGISNET with ArcInfo |
| --- | --- | --- |

Running SINMAP alone produce loose of external border information as well alone channels. This is due to a mistake in the SINDEX.C, C code that does not avoid the division by 0. And fail in the wetness parameter when w1 = x1*a/SS or when w2 = x2*a/ss is divided by 0. This mistake is showed the oval. The square show a zoom in the following figure, to show how the data are similar and different from SINMAP and LOGISNET.

| 1)SINMAP. | b)LOGISNET with SINMAP | c) LOGISNET with ArcInfo |
| --- | --- | --- |

| The variation in values between SINMAP and LOGISNET with SINMAP are in a magnitude between a one one-hundred thousand and one one-million thousand part. Between SINMAP and LOGISNET with Arcinfo inputs this difference go for 1 decimal to centesimal parts. |  |
| --- | --- |

Running SINMAP-Modified have several advantages:

- It takes in account Soil density, because the variable soil density is no longer taken as variable, but as map. That means that each sub region can have different soil density.

- The geotechnical parameters are not limited to just one calibration area (one Region) or to a multiple calibration area (several Sub-regions) inputs. Now it takes in account point samples. Each point-sample can have its own properties. But also each point-sample can have one or several layer-bed with its own geotechnical properties.

| On SINMAP. The calibration area, with multiple regions ( the fields soildensity, waterdensity, bedthick, and weight. Are generated by LOGISNET, no by SINMAP) |  |
| --- | --- |
| LOGISNET. Layer-beds geothecnical parameters are related to a sample-points and in turn each sample-point is related to an a region. The database table is given by the user as a ASCII file with extension TXT. Must contain the following 16 parameters: 1) An unique and consecutive Id #, 2)region Id #, 3)region name, 4) Sample-point-id, 5)layer-bed-id, 6)gravity constant, 7)water density, 8)soil density, 9)lower t/r , 10)Upper t/r , 11)lower cohesion, 12)upper Cohesion ,13)lower friction angle, 14) upper friction Angle 15) layer-bed-thickness, 16)user weight. The file is imported in a INFO table called PARAMETERS. This is because the point cover is not allow to have in its database a relation 1 to many. |  |

HOW TO GO FROM LAYER BEDS TO A REGION SAFETY FACTOR?

Under LOGISNET three alternatives ways were developed, each one with 3 sub-options:

I)Stability Index Multiple Global

1)Stability Index Using Global Average of Sampling Points Per Region And No Weighted (DEFAULT)

2)Stability Index Global Average of Sampling Points Per Region Weighted Per Bed Thickness

3)Stability Index Using Global Average of Sampling Points Per Region using User Weight"

II)Stability Index Multiple Per Sample Point

1)Stability Index Using Point Average of Sampling Points Per Region And No Weighted

2)Stability Index Using Point Average of Sampling Points Per Region Weighted Per Bed Thickness"

3)Stability Index Using Point Average of Sampling Points Per Region using User Weight

III)Stability Index Multiple Per Layer Bed

1)Stability Index Using Each Layerbed of Sampling Points Per Region And No Weighted

2)Stability Index Using Each Layerbed of Sampling Points Per Region Weighted Per Bed Thickness

3)Stability Index Using Each Layerbed of Sampling Points Per Region using User Weight

The first one, the Stability Index Multiple Global was developed for a quick analysis. This option car run the model in the same fashion as SINMAP does by put in a ASCII file a one data points for each region. Each data point sample with just one bed layer. The ASCII file will have the following information: Unique Id, region-id, region-name, sample-id, layer-bed-id, water-density, soil-density, t/q-min, t/q-max, c-min, c-max, phi-min,phi-max, soil-thickness and user-weight:

1,      1,      'REGION1',      1,      1,      9.81,      1000.0,      2000.0,      2000.0,      3000.0,      0.0,       0.25,     30.0,      45.0,      1

2,      2,      'REGION2',      1,      1,      9.81,      1000.0,      1200.0,      2000.0,      2500.0,      0.12,     0.25,     30.0,      45.0,      1

3,      3,      'REGION3',      1,      1,      9.81,      1000.0,      2000.0,      2000.0,      3000.0,      0.0,      0.25,      30.0,      45.0,      1

4,      4,      'REGION4',      1,      1,      9.81,      1000.0,      2000.0,      2000.0,      3000.0,      0.0,       0.25,     32.0,      55.0,      1

Also each sample point will can have 1 or more bed-layer each one with its own geotechnical properties. The file will looks like:

Each column separate by a comma and not head line. In a graphic way this will looks like:

| The calibration area, with 4 regions | The associate info file name.VAT will looks like: After import the data set to LOGISNEt will looks like: |
| --- | --- |
| The sample-point cover with n points: | The associate name.PAT info file will looks like: |
| The data set shows4 regions. Region 1 with 4 sapling points: Region Sampling Points # of layer-beds 1 1 4 1 2 3 1 3 2 1 4 1 | POINT1 with 4 layer-beds POINT2 with 3 layer-beds POINT3 with 2 layer-beds POINT4 with 1 layer-beds |
| Region2: | POINT1 with 3 layer-beds POINT2 with 2 layer-beds POINT3 with 1 layer-beds |
| Region3 | POINT1 with 4 layer-beds POINT2 with 2 layer-beds |
| Region4 | POINT1 with 1 layer-beds |

The Stability Index Multiple Global method, what it does is average all the data set and obtains the mean for each region. In the previous example, the model will run with the following 4 average region parameters:

After the data table is shrinking by using inGeotehcnicalpoint−sampleparametersateachregioninsamplepointsateachregion for each region, the model of stability is activated and run to generate a unique map.

Since the beginning, when the user runs the calibration option, the shrink procedure take place. An ASCII file called STATITIC.TXT is generate. The file contain 18 parameters: 1)Region Name,2) Frequency (max # of points at each region), 3)Region ID (no pay attention to the decimal), 4)Gravity Constant mean, 5)Water Density mean, 6)Soil Density mean , 7)lower t/r mean, 8)Upper t/r mean, 9)lower Cohesion mean, 13)Upper Cohesion mean,14)lower Friction Angle mean, 15) Upper Friction Angle mean 16) LayerBedThickness mean, 17)maximum sample-id (not pay attention to the decimals) and, 18)maximum layer-bed-id (not pay attention to the decimals). Eg.

'REGION1',     10,    1.0,     9.81,    1000.0,     2000.0,     1900.0,     2900.0,     0.0,     0.25      27.0,     40.5,     33.0,     4.0,     4.0

'REGION2',     6,     2.0,     9.81,     1000.0,     2000.0,     1833.33,   2833.33,   0.05,   0.26,     30.0,     45.0,     60.0,     3.0,     3.0

'REGION3',     6,     3.0,     9.81,     1000.0,     2000.0,     2000.0,     3000.0,     0.0,     0.25,     30.0,     45.0,     12.0,     2.0,     4.0

'REGION4',     1,     4.0,     9.81,     1000.0,     2000.0,     2000.0,     3000.0,     0.0,     0.25,     32.0,     55.0,      1.0,      1.0,     1.0

WHAT YOU GET By running LOGISNET on this option, only the final stability index is saved. A TMP subdirectory is temporal created to run the model and is erased after the sequence is ended. Important Note. During all the procedures, The calibration cover grid and its associate INFO database name.VAT are modified to run the model. However, after all the manipulation end, the calibration cover grid is recovered to its original value (the one that have when was created with the create a calibration region module).

The sub options Weighted Per Bed Thickness, what it does is calculate the total thickness for each sample-point, divided each layer-bed thickness per the total thickness and multiply the resulting weight value per all the columns in the data set. Ones the weight is calculated, then the table is shrinking and the model is run. The default option is weight all the columns in the table, but the user can choose which column will be affected.

When only one column is choosed to be weighted. The model runs as follows:

- Calculate the total thickness for each sample-point, divided each layer-bed thickness per the total thickness, and multiply the resulting weight value per the selected column in the data set. Ones all layer-bed of the selected column are weighted then are averaged all per region.

- For those columns no weighted the original value that was calculated since the calibrarion module is leaved thers. (becase is already the average of all bed-layers belonging  to n region)

Importan Note. Weight by soildensity, will no bring any change, because the weight also take place over water density. An even the weight produce a decrement in value this is proportional. It was done in this way becaasue SINMAP model have a ratio r = waterdensity/ soildensity. If I change soildensity and not water density the model produce an unspected result. Black spot on the map with value aout of ranges of SINMAP. Al other parameters works well

WHAT YOU GET. Beside the stability index map nameSI created under the working directory you will get by running LOGISNET on this option, two temporal subdirectories called TMP and TMP2. The TMP subdirectory is to temporal generated to run the model. The subdirectory is erased after the sequence is ended. The TMP2 subdirectory is generated to hold extra information. (The user is ask to save this information or not after the model finish to run). In this case TMP2 will contain: 1) A copy of the calibration nameCAL  grid cover. 2)the writhed calibration nameCAL2 grid cover. 3)A copy of the final Stability Index map nameSI, 4)A copy of the sample-point cover, 5)A copy of the user geotechnical parameters.TXT, 6)A copy of the statistic.txt file, 7)The original INFO PARAMETER and, 8)The weighted parameters in INFO file called PARAMETERS2.

The sub options Weighted by User Weight, what it does is multiply the user weight value per all the columns in the data set. The weight was given by the user in the importing table. The user is responsible to check that thee sum of weight for each point be 1. Ones the weight is calculated, then the table is shrinking and the model is run. The default option is weight all the columns in the table, but the user can choose which column will be affected.

WHAT YOU GET. Same as the sub option Weighted Per Bed Thickness.

Be aware that run the model by average parameter may not represent the best representation of geotechnical parameters in a region. However, is good for a quick sample point on a regional scale, where few samples are been taken.

Important note: On logisnet all calculation of stability index are made them on two temporal subdirectories called TMP and TMP2. The first one, TMP subdirectory is to generate all the partial map functions needed to run the model, and is erased after the sequence is ended. The TMP2 subdirectory is generate when the model run as appoint model or as a bed-layer model and can be holded by the user at the end by saying save in the menu that is presented to him after the model finish.

II)Stability Index Multiple Per Sample Point

The sub option Stability Index Multiple per Sample Point what it does is inGeotehcnicallayer−bedparametersateachsample−pointsinlayer−bedineachsample−points

The used parameters to run this option are shows in the following figure.

What it does is calculate the total thickness for each sample-point, divided each layer-bed thickness per the total thickness and multiply the resulting weight value per all the columns in the data set. Ones the weight is calculated, then the table is shrinking per point-sample and the model is run. In other words the model runs with the sum of all layer-beds contained in each point. For example, ones the weight is calculate and and the bed layer are added

This method delay in calculation as the maximum number of points-sample founded in of all the regions.

WHAT YOU GET. Beside the stability index map nameSI created under the working directory you will get by running LOGISNET on this option, two temporal subdirectories called TMP and TMP2. The TMP subdirectory is to temporal generated to run the model. The subdirectory is erased after the sequence is ended. The TMP2 subdirectory is generated to hold extra information. (The user is ask to save this information or not after the model finish to run). In this case TMP2 will contain:

1) A copy of the calibration nameCAL  grid cover.

2)A copy of the final Stability Index map nameSI,

3)A copy of the sample-point cover,

4)A copy of the user geotechnical parameters.TXT,

5)A copy of the statistic.txt file, 6)The original INFO PARAMETER and,

6)The weighted parameters in INFO file called PARAMETERS2,

7) ASCII files named 1statregpoints1.txt to 1statregpointsN.txt. The file are a temporal file that has a reselection of the data base table with the needed parameters per point to run the model. The files are used to generate the average statistics for each point and generate the files named 2statregpoints1.txt to 2statregpointsN.txt, this files replace the information of nameCAL2 and are used to calculate the final map. In our example we ended with:

4 files called : 1statregpoints1.txt, 1statregpoints2.txt, 1statregpoints3.txt and, 1statregpoints4.txt

8) ASCII files named 2statregpoints1.txt to 2statregpointsN.txt. This files contain the parameters needed to run the model and generate grids that goes from name1 to nameN. This is because the model runs as the number of maximum number of points in all the regions.  In our example will run 4 times. Four grid maps number form name1 to name4 are generated. Each one represent running the model for all points1, all points 2 all points 3 all point4. This was done to increase the speed on computation. In our example we ended with 4 files named:

4 files called : 2statregpoints1.txt, 2statregpoints2.txt, 2statregpoints3.txt and, 2statregpoints4.txt

9)Grid cover name1 to nameN. This grids are represent running the model for each point and are used to calculate final nameSI map. In our example we ended with 4 files named: hola1, hola2, hola3 and, hola4.

What is the logic to generate all these partial files and end with a unique SI map. Let’s use our example. Base on the table we can observe that we have 10 sample points (4 for region1, 3 for region2, 2 for region3 and one for region4). That implied  run 10 times the SAT or the SI model. However, to increase computational speed, we run the model not per point but per area. This allow to run SAT or SI just 4 times. To do that, first we extract all points ones of all our regions. Each of this files have the following information:

Id, regid, regname, sampleid, layerbedid, waterdensity,soildensity, t/qmin, t/qmax, cmin, cmax, phimin,phimax, soilthickness. The files will look like:

1statregpoints1.txt

1,1,'REGION1',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

2,1,'REGION1',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

3,1,'REGION1',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

4,1,'REGION1',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

11,2,'REGION2',1,1,9.8100,1000.0000,2000.0000,2000.0000,2500.0000,0.1200,0.2500,30.0000,45.0000,100.0000

12,2,'REGION2',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.3500,30.0000,45.0000,20.0000

13,2,'REGION2',1,3,9.8100,1000.0000,2000.0000,2000.0000,3500.0000,0.1800,0.2500,30.0000,45.0000,100.0000

17,3,'REGION3',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

18,3,'REGION3',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,12.0000

19,3,'REGION3',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

20,3,'REGION3',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

23,4,'REGION4',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,32.0000,55.0000,1.0000

1statregpoints2.txt

5,1,'REGION1',2,1,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,10.0000,25.0000,10.0000

6,1,'REGION1',2,2,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,20.0000,30.0000,100.0000

7,1,'REGION1',2,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,35.0000,10.0000

14,2,'REGION2',2,1,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

15,2,'REGION2',2,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000

21,3,'REGION3',2,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

22,3,'REGION3',2,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

1statregpoints3.txt

8,1,'REGION1',3,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,15.0000

9,1,'REGION1',3,2,9.8100,1000.0000,2000.0000,3000.0000,4000.0000,0.0000,0.2500,30.0000,45.0000,15.0000

16,2,'REGION2',3,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

1statregpoints4.txt

10,1,'REGION1',4,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000

Once the resection has been done, the average of all layer-bed for each point is calculated, to generate the files 2statregpoints1.txt to 2statregpointsN.txt. Each of these files has the following information: On the first line a header title with: region-name, frecuency (# of beds), region-id, water-density, soil-density, t/q-min, t/q-max, c-min, c-max, phi-min, phi-max, soil-thickness. From the second line to further are the needed parameters separated by comma. The files will look like:

2statregpoints1.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     4,        1,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         80

REGION2,     3,        2,      9.810,       1000,       2000,       2000,       3000,      0.100,      0.283,          30,         45,        220

REGION3,     4,        3,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         52

REGION4,     1,        4,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          32,         55,      1.000

2statregpoints2.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     3,        1,      9.810,       1000,       2000,   1333.333,   2333.333,          0,      0.250,          20,         30,        120

REGION2,     2,        2,      9.810,       1000,       2000,       1500,       2500,          0,      0.250,          30,         45,        120

REGION3,     2,        3,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         20

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

2statregpoints3.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     2,        1,      9.810,       1000,       2000,       2500,       3500,          0,      0.250,          30,         45,         30

REGION2,     1,        2,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,     20.000

REGION3,     0,        3,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

2statregpoints4.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     1,        1,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,    100.000

REGION2,     0,        2,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION3,     0,        3,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

Each of this files represent the parameters on which the model SAT or SI are calculated. After the model runs, 4 files are generated:

| HOLA1 | HOLA2 |
| --- | --- |
| HOLA3 | HOLA4 |

From this partial maps, how we generate the final map?. The answer is by average. In our example the region 4 was run 4 times because have 4 points. Region 2 was run 3 times because its maximum mumbre o fpoints is 3. Region 3 was run 2 times because have 2 points and region 4 was run 1 time because have 1 point.

What we do is generate a temporal file in this way for each region:

TMPHOLA1 = (HOLA1 + HOLA2 + HOLA3 + HOLA4) / 4

TMPHOLA2 = (HOLA1 + HOLA2 + HOLA3) / 3

TMPHOLA3 = (HOLA1 + HOLA2) / 2

TMPHOLA4 = (HOLA1) / 1

Note: When average the map we ended with extra information for other regiosn. For instance, the operation

TMPHOLA1 = (HOLA1 + HOLA2 + HOLA3 + HOLA4) / 4, end with the wright answer for region 1, because have 4 points, but not for all the other region. That is not a problem because further we extract just the region 1 and leave 0 to all others. The same happen when we run TMPHOLA2 = (HOLA1 + HOLA2 + HOLA3) / 3

The operation is the write for region 2, but all others are wrong, and so on.

The final step to get the final map is add all in one map.

- HOLASI := con(HOLACAL.value eq 1, TMPHOLA1, 0) . What state here create a temporal grid file called HOLASI, with the following information: If the calibration area is one, then put the value of my first map TMPHOLA1, otherwise put cero.

- HOLASI := con(HOLACAL.value eq 2, TMPHOLA2, HOLASI) . What state here create a temporal grid file called HOLASI, with the following information: If the calibration area is two, then put the value of my second map TMPHOLA2, otherwise put previous value contained on HOLASI.

- HOLASI := con(HOLACAL.value eq 3, TMPHOLA3, HOLASI) . What state here create a temporal grid file called HOLASI, with the following information: If the calibration area is three, then put the value of my third map TMPHOLA3, otherwise put previous value contained on HOLASI.

- HOLASI = con(HOLACAL.value eq 4, TMPHOLA4, HOLASI) . What state here createthe final grid model file called HOLASI, with the following information: If the calibration area is four, then put the value of my four map TMPHOLA4, otherwise put previous value contained on HOLASI (in which case have all other regions 1, 2 and 3).

The Stability Index Multiple per Sample Point is use full in regions where the user has samples points with layer-bed information, but some of the layer-bed are unknown or has not certainly in all its parameters. The user must be aware that average or weighted the geotechnical properties they may no be the best way to represent the value of one or several geotechnical parameters.

The sub options Weighted Per Bed Thickness, and Weighted by User Weight  do the calculate as was described for the global method, but with weighted  parameters. However, the calculation is per poin sample and not por region. What it does is calculate the total thickness for each sample-point, divided each layer-bed thickness per the total thickness and multiply the resulting weight value per all the columns in the data set. Ones the weight is calculated, then the table is shrinking per point-sample and the model is run. In other words the model runs with the sum of all layer-beds contained in each sample-point. The default option is weight all the columns in the table, but the user can choose which column will be affected. When only one column is has been choused to be weighted. The model runs as follows:

- Calculate the total thickness for each sample-point, divided each layer-bed thickness per the total thickness, and multiply the resulting weight value per the selected column in the data set. Ones all layer-bed of the selected column are weighted then are averaged all per sample-point.

- For those columns no weighted the original value give by the user is leaved and average for each sample-point.

For example running the model weighting per bed layer thickness and with the option all colums. The inputs parameters to run this model will be as is showed on the following figure

Running the model weighting the column tq-min will use the following parameters

As you can see, the model run with the add all layer beds for each point for tq-min and with the average for all the other columns. One important difference with running the model with no weight and weight is in the files:

Each of this files have the following information:

Id, regid, regname, sampleid, layerbedid, waterdensity,soildensity, t/qmin, t/qmax, cmin, cmax, phimin,phimax, soilthickness. The files will look like: 1statregpoints1.txt…1statregpointsN.txt, this is due to all calculation of weight and averages are made it on the table PARAMETER, so when is generate the files 1statregpointsN.txt, will show that all layer on each point have sthe same information. That is I repeated the information. That is ok, because the procedure to obtain a unique representative data is obtained by average and its final result is leaved on the files 2statregpoints1.txt to 2statregpointsN.txt as is show in the example:

1statregpoints1.txt

1,1,'REGION1',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

2,1,'REGION1',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

3,1,'REGION1',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

4,1,'REGION1',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

11,2,'REGION2',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.1000,0.2833,30.0000,45.0000,100.0000

12,2,'REGION2',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.1000,0.2833,30.0000,45.0000,20.0000

13,2,'REGION2',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.1000,0.2833,30.0000,45.0000,100.0000

17,3,'REGION3',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

18,3,'REGION3',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,12.0000

19,3,'REGION3',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

20,3,'REGION3',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

23,4,'REGION4',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,32.0000,55.0000,1.0000

1statregpoints2.txt

5,1,'REGION1',2,1,9.8100,1000.0000,2000.0000,1083.3333,2333.3333,0.0000,0.2500,20.0000,30.0000,10.0000

6,1,'REGION1',2,2,9.8100,1000.0000,2000.0000,1083.3333,2333.3333,0.0000,0.2500,20.0000,30.0000,100.0000

7,1,'REGION1',2,3,9.8100,1000.0000,2000.0000,1083.3333,2333.3333,0.0000,0.2500,20.0000,30.0000,10.0000

14,2,'REGION2',2,1,9.8100,1000.0000,2000.0000,1833.3333,2500.0000,0.0000,0.2500,30.0000,45.0000,20.0000

15,2,'REGION2',2,2,9.8100,1000.0000,2000.0000,1833.3333,2500.0000,0.0000,0.2500,30.0000,45.0000,100.0000

21,3,'REGION3',2,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

22,3,'REGION3',2,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

1statregpoints3.txt

8,1,'REGION1',3,1,9.8100,1000.0000,2000.0000,2500.0000,3500.0000,0.0000,0.2500,30.0000,45.0000,15.0000

9,1,'REGION1',3,2,9.8100,1000.0000,2000.0000,2500.0000,3500.0000,0.0000,0.2500,30.0000,45.0000,15.0000

16,2,'REGION2',3,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

1statregpoints4.txt

10,1,'REGION1',4,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000

2statregpoints1.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     4,        1,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         80

REGION2,     3,        2,      9.810,       1000,       2000,       2000,       3000,      0.100,      0.283,          30,         45,        220

REGION3,     4,        3,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         52

REGION4,     1,        4,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          32,         55,      1.000

2statregpoints2.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     3,        1,      9.810,       1000,       2000,   1083.333,   2333.333,          0,      0.250,          20,         30,        120

REGION2,     2,        2,      9.810,       1000,       2000,   1833.333,       2500,          0,      0.250,          30,         45,        120

REGION3,     2,        3,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,         20

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

2statregpoints3.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     2,        1,      9.810,       1000,       2000,       2500,       3500,          0,      0.250,          30,         45,         30

REGION2,     1,        2,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,     20.000

REGION3,     0,        3,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

2statregpoints4.txt

REGION    FRECUENCY REGIONID GRAVITY  WATER DENSITY SOIL DENSITY   tqmin       tqmax        cmin        cmax       phimin     phimax    bedthick

REGION1,     1,        1,      9.810,       1000,       2000,       2000,       3000,          0,      0.250,          30,         45,    100.000

REGION2,     0,        2,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION3,     0,        3,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

REGION4,     0,        4,          0,          0,          0,          0,          0,          0,          0,           0,          0,          0

The sub options Weighted by User Weight, do the same as weight per layer-bed thickness but using the user weight.

EXTRA IMPORTAN NOTICE: ON CMIN-CMAX. THE MODEL FAIL AN D MAY BE IN OTHER PONDERATE OPTION DUE TO THE CONDITION OF ARCINFO -999. THAT MEAN NO DATA. SON IN THE PARTIOAL MAPS IF SOME OF THEM HAVE -999, WHEN THE OPERATION OF ADD AND AVERAGE TAKE PLACE, ARCINFO PUT NODATA.

IN THE FOLLOWIN EXAMPLE BY WEIGHTING CMIN AND C-MAX THE PARCIAL LOOKS LIKE:

| HOLA1 | HOLA2 |
| --- | --- |
| HOLA3 | HOLA4 |
| HOLASI One solution is replace -999 per 0 and do the addition or average. However more careful check must be done on what the model do. May this paleative result do bad things. This is a reason why I did not correct. And just put the warning to the user. In a second version I will try to correct. One good sign of this mistake is because on LOGISNET will apperar this areas as black spots, and with values out or range (out of 0 to 10). |  |

III)Stability Index Multiple Per Layer Bed

This is the slower method, because run as the maximum number of layer-bed founded in all regions. As in the previous cases. However, some differences are between the no weighted and weighted option. The no weighted option is the more accurately. This is due to limitation on the grid procedure to weight. In a no weighted form, the model is calculate for each layer-bed, then averages by point and then average by region. In the weighted form, the model is run first; the resulting grid maps are then weighted. This procedure was done due to if we weight first then run the model and then add (instead of average) the model SI get estrange result with black spot containing information out of the range that one must expect to have.

The model run with the following parameters, for our example:

The file is imported in a INFO table called PARAMETERS (because the point cover is not allow to have in its database a relation 1 to many.)

The statistics of each region are showed as follow:

| What is the logic to generate all these partial files and end with a unique SI map? . Let’s use our example. Base on the table we can observe that we have 23 different layer-beds . That implied run 23 times the SAT or the SI model. However, to increase computational speed, we run the model not per point but per area. This allow to run SAT or SI just 10 times. To do that, first we extract all layer-bed per points of all our regions. This is done by Select all layer-bed id =1 of all points 1 in all the regions, then run the model, then select all layerbedid = 2 for all points 1 then run the model and so on, when I finish with the points one, I follow with the points two: Select all layerbes id =1 of all points 2 in all the regions, then run the model, then select all layerbedid = 2 for all points 2 then run the model and so on. The files are created under the subdirectory tmp and are called: POINT%I%BED%J%. In this case the files are:: |  |
| --- | --- |

Each of this files heve the following information:

Id, region-d, region-name, sample-id, layerbed-id, water-density, soil-density, t/q-min, t/q-max, c-min, c-max, phi-min, phi-max, soil-thickness:

POINT1BED1.TXT

1,1,'REGION1',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

11,2,'REGION2',1,1,9.8100,1000.0000,2000.0000,2000.0000,2500.0000,0.1200,0.2500,30.0000,45.0000,100.0000

17,3,'REGION3',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

23,4,'REGION4',1,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,32.0000,55.0000,1.0000

POINT1BED2.TXT

2,1,'REGION1',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

12,2,'REGION2',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.3500,30.0000,45.0000,20.0000

18,3,'REGION3',1,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,12.0000

POINT1BED3.TXT

3,1,'REGION1',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

13,2,'REGION2',1,3,9.8100,1000.0000,2000.0000,2000.0000,3500.0000,0.1800,0.2500,30.0000,45.0000,100.0000

19,3,'REGION3',1,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

POINT1BED4.TXT

4,1,'REGION1',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

20,3,'REGION3',1,4,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

POINT2BED1.TXT

5,1,'REGION1',2,1,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,10.0000,25.0000,10.0000

14,2,'REGION2',2,1,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

21,3,'REGION3',2,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

POINT2BED2.TXT

6,1,'REGION1',2,2,9.8100,1000.0000,2000.0000,1000.0000,2000.0000,0.0000,0.2500,20.0000,30.0000,100.0000

15,2,'REGION2',2,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000

22,3,'REGION3',2,2,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,10.0000

POINT2BED3.TXT

7,1,'REGION1',2,3,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,35.0000,10.0000

POINT3BED1.TXT

8,1,'REGION1',3,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,15.0000

16,2,'REGION2',3,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,20.0000

POINT3BED2.TXT

9,1,'REGION1',3,2,9.8100,1000.0000,2000.0000,3000.0000,4000.0000,0.0000,0.2500,30.0000,45.0000,15.0000

POINT4BED1.TXT

10,1,'REGION1',4,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000

With the previous files, the model runs. Afeter the model SAT or SI run a series of grid files with the same name are generated. In our example, this are:

| Each file represents one running for the model. After running the model SAT or SI, another 10 grid cover are created. The name is SIPERPOINT%I%BED%J% . for SI modelt and SATPERPOINT%I%BED%J%. This files the represent the running of layer-bed by layer-bedc. Importat note: The generated files have extra information. Even I put 0 in areas where no information is available. For example: POINT4BED1.TXT 10,1,'REGION1',4,1,9.8100,1000.0000,2000.0000,2000.0000,3000.0000,0.0000,0.2500,30.0000,45.0000,100.0000 Just region 1 have information so I put cero to region 2, 3, 4. The model put 1 after run or maybe another value. That doses no t affect the final result, because the only that must be do is put a condition that say that if is on area1 put the value of this region otherwhise put 0. |  |
| --- | --- |

| SIPOINT1BED1 | SIPOINT1BED2 |
| --- | --- |
| SIPOINT1BED3 | SIPOINT1BED4 |
| SIPOINT2BED1 | SIPOINT2BED2 |
| SIPOINT2BED3 | SIPOINT3BED1 |
| SIPOINT3BED2 | SIPOINT4BED1 |

| With the previous files, a new average is generated to represent points per regions. This is done by average all points per region. The files called REG%J%POINT%I% in our example this files. are: - |  |
| --- | --- |

So for instance for region 1, because have 10 layer-beds in all its 4 points  the calculation is

REG1POINT1 = (SIPOINT1BED1 + SIPOINT1BED2 + SIPOINT1BED3 + SIPOINT1BED4) /4

REG1POINT2 = (SIPOINT2BED1 + SIPOINT2BED2 + SIPOINT2BED3) / 3

REG1POINT3 = (SIPOINT3BED1 + SIPOINT3BED2) / 2

REG1POINT4 = (SIPOINT4BED1) / 1

And all other region are given the value of 0.

For regione2:

REG2POINT1 = (SIPOINT1BED1 + SIPOINT1BED2 + SIPOINT1BED3) / 3

REG2POINT2 = (SIPOINT2BED1 + SIPOINT2BED2) / 2

REG2POINT3 = (SIPOINT3BED1) / 1

For regione3:

REG3POINT1 = (SIPOINT1BED1 + SIPOINT1BED2) / 2

REG3POINT2 = (SIPOINT2BED1 + SIPOINT2BED2) / 2

REG3POINT3 = (SIPOINT3BED1) / 1

REG3POINT4 = (SIPOINT4BED1) / 1

For region4

REG1POINT1 = (SIPOINT1BED1) / 1

In all this cases the covers are average, because they are row data.  When weighted are used, then instead of average, we just add, this is because the SI or SAT model is already weighted. How this happen? The model run and is multiply per its weight  so for instance after the first run the SI map ended with

In other words, in the case of weighted due to all the SIPOINT%I%BED%J% are weighted (from the si1perbed2.aml, with the option: ..\tmp2\siPoint%b%Bed%a% = ..\tmp2\siPoint%b%Bed%a%  *  ..\%.sinmapdem2%cal.weigth. the only thing that is done is add (REG1POINT1 = (SIPOINT1BED1 + SIPOINT1BED2 + SIPOINT1BED3 + SIPOINT1BED4)

SIPOINT1BED1 * = Temporal REG1POINT1

SIPOINT1BED2 * = Temporal REG1POINT1

SIPOINT1BED3 * = Temporal REG1POINT1

SIPOINT1BED4 * = Temporal REG1POINT1

The addition of all partial Temporal REG1POINT1 = FINAL REG1POINT1

The final stem is go from points to a region. This is done by average all the files REG%j%POINT%i%  that belong to that region. In our example 4 cover are generated :

REG1 = (REG1POINT1 + REG1POINT2 + REG1POINT3 + REG1POINT4 )/ 4

REG2 =  (REG2POINT1 + REG2POINT2 + REG2POINT3 )/ 3

REG3 =  (REG3POINT1 + REG2POINT2)/ 2

REG4 =  (REG4POINT1)/ 1

The final step to get the final map is add all in one map.

HOLASI = REG1 +REG2 + REG3 + REG4    FINAL STABILITY MAP